DECLARATION OF PERFORMANCE

| No: | 039694520059 |
| --- | --- |
| 1. Unique identification code of the product-type: | Bauder JFRI Upstand Insulation Grades EPS 200 |
| 2. Type, batch or serial number or any other element allowing identification of the construction product as required pursuant to Article 11(4) of the CPR: | Bauder JFRI Upstand Insulation |
| 3. Intended use or uses of the construction product, in accordance with the harmonised technical specification, as foreseen by the manufacturer: | EPS (uniform thickness) insulation with a 9mm magnesium based board finish for use in vertical upstand detail protection for inverted flat roofs |
| 4. Name, registered trade name or registered trade mark and contact address of the manufacturer as required pursuant to Article 11(5): | Bauder Ltd 70 Landseer Road Ipswich Suffolk IP3 0DH |
| 5. Where applicable, name and contact address of the authorised representative whose mandate covers the tasks specified in Article 12(2): | Not Applicable |
| 6. System or systems of assessment and verification of constancy of performance of the construction product as set out in Annex V: | AVCP System 3 |
| 7. In case of the declaration of performance of the construction product covered by a harmonised standard: - name and number of notified body: | British Board of Agrément; BRE (RtF) 9mm magnesium board finish not covered by harmonised standard. Third party material characterisation by CERAM Ref:132179 |
| - performed: | Reaction to Fire Thermal Conductivity Compressive Strength |
| - under system: | System 3 |
| - and issued: | Test report on application |

Declared Performance

| Essential Characteristic | Performance | Harmonised technical standard |
| --- | --- | --- |
| Reaction to Fire | Euroclass E | BS EN 13501-1:2007 |
| Durability of RtF against ageing / degradation | Fire performance of EPS does not deteriorate with time | BS EN 13163:2012 |
| Reaction to fire (9mm board) | A1 Non-Combustible | BS EN 13501-1:2007 |
| Thermal Conductivity - Declared | 0.033W/mK | BS EN 13163:2012 |
| Thermal Resistance | See table 3 | BS EN 13163:2012 |
| Compressive Strength at 10% deformation | 200 kN/m2 | BS EN 12086:2013 |
| Dimensional stability | DS(70,90)1 | BS EN 1604:1997 |
| Long-term water absorption by diffusion | WD(V)3 | BS EN 12088:1997 |
| Long-term water absorption by immersion | WL(T)2 | BS EN 12087:1997 |
| Compatibility with other components | On application | - |

Thermal Resistance

| Nominal Board Thickness | Thermal Resistance m2K/W |
| --- | --- |
| 59 | 1.50 |
| 84 | 2.20 |
| 109 | 2.95 |

The performance of the product identified in points 1 and 2 is in conformity with the declared performance.

This declaration of performance is issued under the sole responsibility of the manufacturer identified in point 4.

On behalf of the manufacturer by:

Paul Felgate, R&D Manager	1st May 2014